UHF Prime Reader

User Manual V1.2

Table of contents

Table of contents	1

1. Docking Communication Protocol	2

1 .1 Serial communication parameters	2

1 .2 Data frame format	2

2 Command Frame Set	4

2.1 Command List	4

2.2 Module general control commands	5

2.2.1​ Initialize device 	5

2.2.2​ Device factory reset 	6

2.2.3​ Set RF output power ......................................	6

2.2.4​ Set / read the RF protocol standards supported by the module	7

2.2.5  Set / Read Device Network Port Information	8

2.2.6​Set / read device remote network port information	9

2.2.7​ Get module information	11

2.2.8  Set all configurable parameters	12

2.2.9​ Read all configurable parameters	18

2.2.10​Set read permission	19

2.2.11​Set read input and output control	20

2.2.12  Set/Read WiFi Information	22

2.2.13​Set read permission	23

2.2.1 4  Release/Close Relay 1	24

2.2.1 5  Release/Close Relay 2	26

2.2.16​ Set the RSSI filter threshold for reading each antenna port	27

2.2.17​ Set LED Status	28

2.3 International standard (ISO 18000-6 C ) protocol related commands	29

2.3.1  Label Inventory 	29

2.3.2  Stop Inventory	31

2.3.3  Read tag data 	31

2.3.4​ Write tag data	33

2.3.5  Lock tag data	35

2.3.6  Kill tag	37

2.3.7​ Select the tag to be operated	38

2.3.8​ Set Select command parameters	39

2.3.9​ Get Select command parameters	40

2.3.10​Get Query command parameters	42

2.3.11​Set Query command parameters 	43

2.4 MQTT related commands	44

2.4.1  JSON message body	44

2.4.2  Set/Read MQTT connection information	45

2.4.3  Set/Read serial data topic information	46

2.4.4  Set/Read Published Serial Data Topic Information	48

2.4.5  Set/Read Publish Tag Data Topic Information	50

2.4.6  Set/Read Publish IO Event Topic Information	51

Appendix A. Operational Status Returned by Tags	53

Appendix B. CRC16 checksum reference C code	53

Appendix C. Definition of STATUS.	54

1. Docking Communication Protocol

The module can communicate with external applications via serial ports RS 232 / TTL , RS485 and network port RJ45 .

1 .1 Serial communication parameters

The module communicates with external applications through the serial port

Table A-1 Module connection serial port parameters

| project | parameter | Remark |
| --- | --- | --- |
| Baud rate | 115200bps |  |
| Data bits | 8 |  |
| Stop bits | 1 |  |
| Parity bit | None |  |

1 .2 Data frame format

The data packet sent by the upper layer application is called "command" below, and the data packet returned by the module to the upper layer application is called "response" below. The length unit of all the following data segments is bytes.

Data transmission order: For each data item consisting of multiple bytes, the most significant byte is sent first, and the least significant bit is sent first within the byte.

The data frame format of the command is shown in Table A-2:

Table A-2 Command data frame format

| MSB |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- |
|  | Control Field |  |  | Information Field | EpLENogue Field |
| HEAD | ADDR | CMD | LEN | Data[] | CHECK |
| 1 Byte | 1 Byte | 2Bytes | 1 Byte | 0~255Byte | 2 Byte |

The response data frame format is shown in Table A-3:

Table A-3 Response data frame format

| MSB |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- |
|  | Control Field |  |  | Information Field |  | EpLENogue Field |
| HEAD | ADDR | CMD | LENS​ | STATUS | Data[] | CHECK |
| 1 Byte | 1 Byte | 2Bytes | 1 Byte | 1 Byte | 0~254Byte | 2 Byte |

The data frame structure consists of the following three parts: frame control field (Control Field), information data field (Information Field) and frame tail field (EpLENogue Field).

|  |  | length (byte) | illustrate |
| --- | --- | --- | --- |
| Control Field | HEAD​ | 1 | HEAD is fixed as the hexadecimal number 0x CF , which is used for data frame synchronization. |
|  | ADDR​ | 1 | Reader address. Address range: 0x00~0xFE, 0xFF is the broadcast address, the reader only responds to commands with the same address as itself and 0xFF. The address of the reader is 0x00 when it leaves the factory. |
|  | C MD | 2 | Command code and command word definitions are shown in Table A-7 . |
|  | LENS​ | 1 | This length is the length of all bytes of the actual Information Field . The maximum data length is 255 bytes ( 0xFF ). |
| Information Field | S TATUS | 1 | STATUS is the execution status of the host computer command contained in the module's response. STATUS only exists in the response data frame returned by the module. There is no STATUS part in the command frame sent by the upper application. STATUS is 0 , which means that the command is executed successfully. Here, successful execution only means that the module successfully receives the tag response. If there is a tag execution status in the tag response, it is necessary to further determine whether the tag execution status is correct. The definition of STATUS is shown in Table A-6 . |
|  | Data[] | indefinite | Parameter field. In actual commands, it may not exist. Data[ ] is the actual data to be transmitted. The valid bytes of Data[ ] will be specifically defined in each command format . |
| EpLENogue Field | MSB -CRC16 | 1 | CRC16 high byte. CRC16 is the CRC16 value from HEAD to Data[] . |
|  | LSB -CRC16 | 1 | CRC16 low byte. |

See Appendix B for the CRC16 reference code .

2 Command Frame Set

The UHF RFID reader/writer module is referred to as "module" in the following description.

2.1 Command List

Table A-7 Control command example list

| Command Name | CMD control byte | Functional Description | Remark |
| --- | --- | --- | --- |
| EPC Directive |  |  |  |
| RFM_INVENTORYISO_CONTINUE | 0x00 01 | Label inventory ( process ) |  |
| RFM_INVENTORYISO_STOP | 0x00 02 | Stop label counting |  |
| RFM_READISO_TAG | 0x0003 | Read tag data |  |
| RFM_SETISO_SELECTMASK | 0x0007 | Set the label EPC number |  |
| RFM_SET_SELPRM | 0x00 10 | Select parameter settings |  |
| RFM_GET_SELPRM | 0x00 11 | Select parameter acquisition |  |
| RFM_SET_QUERY_PARAM | 0x00 12 | Query parameter settings |  |
| RFM_GET_QUERY_PARAM | 0x00 13 | Query parameter acquisition |  |
| Module custom directives |  |  |  |
| RFM_MODULE_INT | 0X0050 | Initialize the device | Stop all actions |
| RFM_REBOOT | 0x00 52 | Restore factory settings | Restore Defaults |
| RFM_SET_PWR | 0x0053 | Setting RF Power | 0-26dBm, all values above 26 are 26 |
| RFM_SET_GET_RFIDPRO | 0x0059 | Set/read RF protocol | Currently the device only supports ISO 18000-6C |
| RFM_SET_GET_NETPARA | 0X005F | Set/read network port information | Applicable to LAN |
| RFM_SET_GET_REMOTE_NETPARA | 0x0064 | Set to get the remote network port information | Applicable to the entire network, cross-segment remote |
| RFM_GET_DEVICEINFO | 0X0070 | Get all module version numbers and sn codes |  |
| RFM_SET _ALL_PARAM | 0x0071 | Set all basic parameters | Package all basic parameters and set them all at once |
| RFM_ GET_ALL_PARAM | 0x0072 | Read all basic parameters | Package all basic parameters and read them all at once |
| RFM_ SET_GET_PERMISSION_PARAM​​ | 0x0073 | Read permission settings |  |
| RFM_SET_ GET_IOPUT _PARAM​ | 0x0074 | Input and output IO settings |  |
| RFM_SET_GET_WiFi_PARAM | 0x0075 | Set/read WiFi information |  |
| RFM_SET_GET_S- PERMISSION_PARAM | 0X0076 | Specific read permission settings |  |
| RFM_RELEASE_CLOSE_RELAY1 | 0X0077 | Release/Close Relay |  |
| RFM_RELEASE_CLOSE_RELAY2 | 0X0078 | Release/Close Relay |  |
| RFM_SET_GET_AntN-RSSI-Filter | 0x0079 | Set/read each antenna RSSI threshold |  |
| RFM_TEST_LEDCFG | 0x00E9 | Setting up the LED light strip |  |
| MQTT Commands |  |  |  |
| RFM_SET_GET_ MQTT_ConnectInfo | 0x007B | Set/read MQTT_Connect information |  |
| RFM_SET_GET_ Subscribe_SerialData_TopicInfo​​ | 0x007C | Set/read serial data subscription topic information |  |
| RFM_SET_GET_Publish_SerialData_TopicInfo​​ | 0x007D | Set/read serial data publishing topic information |  |
| RFM_SET_GET_Publish _Tag Data_TopicInfo | 0x007E | Set/read release tag data topic information |  |
| RFM_SET_GET_Publish _IOeven _TopicInfo | 0x007F | Set/read published IO event topic information |  |

2.2 Module general control commands

2.2.1​ RFM_MODULE_INT ( Initialize device )

The device is initialized and configured according to the stored parameters.

- Format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x00 5 0 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x 00 | 0x00 5 0 | 0x01 | 1 Byte | 2Byte |

 STATUS: This command will only return a successful execution (value 0x00), other values are invalid;

2.2.2​ RFM_REBOOT ( device factory reset )

Restore factory settings command. When this command is successfully executed, the module will be restored to factory settings (does it need to be restarted?) .

- Format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x00 52 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x 00 | 0x00 52 | 0x01 | 1 Byte | 2Byte |

 STATUS: This command will only return a successful execution (value 0x00), other values are invalid;

2.2.3​​ RFM_SET_ PWR 	( Set RF output power )

This command is used to set the module's RF output power. Before using the module to operate the tag, the user needs to use this command to set the module's RF output power. If the user does not set the module's RF output power, the module will use the default setting when it works.

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Power | Resv |  |
| 0xCF | 0xFF​ | 0x00 53 | 0x02 | 1 Byte | 1 Byte | 2Byte |

  - Power: output power, in dBm, value range: [0, 26 ]dBm, values greater than 26 dBm are 26 dBm .

  - Resv: System reserved field, default value is 0x00;

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 53 | 0x01 | 1 Byte | 2Byte |

  - STATUS:

  - 0x00: command executed successfully;

  - 0x01: The module does not support the output power value;

  - Other values: invalid;

2.2.4​​ RFM_SET_GET_RFID PRO ( Set / read the RF protocol standards supported by the module )

This command sets/reads the module's radio frequency RFID protocol standard specification ( by issuing this command, the protocol standard can be set to determine whether the module is successfully connected ).

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | R FIDPRO |  |
| 0xCF | 0xFF​ | 0x00 59 | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Set , followed by RFIDPRO of 1 Byte in length ;

0x02: Read , then do not connect to RFIDPRO;

Other values: invalid;

  - RFIDPRO : Protocol options , see the table below for details.

- Response format and status byte

Set the response:

| HEAD | ADDR​ | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 59 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | RFIDPRO |  |
| 0xCF | 0x 00 | 0x00 59 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Byte |

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: Command execution successful ; 0x01 : Parameter error. |
| Option | 1 | 0x01: Set; 0x02: Get |
| RFIDPRO | 1 | 0x00: ISO 18000-6C; 0x01: GB/T 29768 ; 0x02: GJB 7377.1 ; Currently only ISO is supported 18000-6C. |

2.2.5 RFM_ SET_GET_NETPARA ( Set / Read Device Network Port Information )

This command sets/reads the module's network port information. The default setting is TCP Service, the host computer connection needs to set the corresponding device IP address and port;

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | NetPara |  |
| 0xCF | 0xFF​ | 0x00 5F | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Setting;

0x02: read;

Other values: invalid;

- Response format and status byte

Set the response:

| HEAD | ADDR​ | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 5 F | 1 Byte | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x 00 | 0x000 5 F | 1 Byte | 1 Byte | 1 Byte | NByte | 2Byte |

| Fields |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: Command execution successful ; 0x01 : Parameter error. |
| Option |  | 1 | 0x01: Set; 0x02: Get |
| NetPara | IPAddr | 4 | Device IP address. If the IP address is 192.168.1.1, the data is [ 0xC0, A8, 0x01, 0x01] |
|  | MacAddr​ | 6 | Mac address, if multiple devices work at the same site, please use different MAC addresses; For example: mac[6]={0x00,0x08,0xdc,0x11,0x11,0x11} , the mac address is 00-08-DC-11-11-11 |
|  | Port | 2 | Listening port, the value is [ 0,65536] , the default is 9527; |
|  | NetMask | 4 | Subnet mask, default is [0xFF,0xFF,0xFF,0x00] |
|  | GateWay | 4 | Default gateway, default is [0xC0,0xA8,0x01,0x01] |

2.2.6​​ RFM_SET_GET_REMOTE_NETPARA (Set / read device remote network port information )

This command sets/reads the remote network port information. The device sends data to the remote IP, and the remote computer can monitor and receive the data.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | NetPara |  |
| 0xCF | 0xFF | 0x00 64 | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Setting;

0x02: read;

Other values: invalid;

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x00 5F | 1 Byte | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x00 | 0x000 5F | 1 Byte | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

| Fields |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option |  | 1 | 0x01: Set; 0x02: Get |
| NetPara | Enable | 1 | Enable switch, 1: enable sending, 0: disable sending. |
|  | IPAddr | 4 | Remote IP address. If the IP address is 192.168.1.1, the data is [0xC0, 0xA8, 0x01, 0x01] |
|  | Port | 2 | Listening port, the value is [0,65536], the default is 9527; |
|  | HeartTime | 1 | Heartbeat packet time, to prevent disconnection, unit*5S |

2.2.7​ RFM_GET_ DEVICE INFO (Get module information)

This command is used to obtain the version information of the current device , including the hardware version number, firmware version number, and SN number of the CP module and RFID module .

- Format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0x CF | 0xFF​ | 0x00 70 | 0x00 | 2 Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CPHardVer | CPFirmVer | CPSN_code |
| 0xCF | 0x00 | 0x0070 | 0xAC | 1Byte | 32Bytes | 32Bytes | 12Bytes |

| PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- |
| RFIDModeVer | RFIDModeName | RFIDMode _SN Code​ |  |
| 32Bytes | 32Bytes | 1 2 Bytes | 2 Byte |

Definition of each byte:

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | This command will only return a successful result (value 0x00), other values are invalid; |
| CP HardVer | 32 | CP module hardware version number , in ASCII format, ending with '\0'. The actual hardware version number may be less than 32 bytes, and the remaining bytes not used are supplemented with '\0'. |
| CP FirmVer | 3 2 | CP module firmware version number and bootloader version number, the format is the same as the hardware version number; |
| CP _SN Code​ | 1 2 | CP module 12-byte serial number, in ASCII format, with unused bytes supplemented with '\0'; |
| RFID Mode Ver | 32 | RFID module version number , in ASCII format, ends with '\0' . The actual version number is 2 bytes, less than 32 bytes, and the unused bytes are supplemented with '\0' . The first byte is the main version, and the second byte is the sub-version; for example: Byte0=0x0, Byte1=0x1e, the version number is 3.30. |
| RFIDModeName | 3 2 | RFID module model , in ASCII format, ending with '\0' . The actual model is less than 32 bytes, and the unused bytes are supplemented with '\0' . |
| RFIDMode _SN Code​ | 1 2 | RFID module serial number, in ASCII format. The actual serial number is 4 bytes, less than 32 bytes. The unused bytes are supplemented with '\0'. |

2.2.8 RFM_SET_ ALL_PARAM ( Set all configurable parameters )

This command is used to set all configurable parameters of the device . Before the user uses the module to operate the tag, it is necessary to use this command to set all configurable parameters of the device . If the user does not make this setting, the device will use the default settings when it works.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF | 0x0071 | 0x19 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Bytes​ |

  - Addr: The communication address of the device, the default value is 0x00. This address cannot be 0xFF. If it is set to 0xFF, the read/write module will return parameter error information.

  - RFIDPRO: Protocol standard specification of radio frequency RFID equipment , 0x00: ISO 18000-6C; 0x01: GB/T 29768 ; 0x02: GJB 7377.1 ; Currently only ISO is supported 18000-6C.

  - WorkMode: The working mode of the device, the default value is 0 .

| WorkMode | Working Mode |
| --- | --- |
| 0 | Answer Mode |
| 1 | Active Mode |
| 2 | Trigger Mode |

  - Interface: The communication interface of the device. The default value is 0x80 . The specific meanings are as follows:

| Interface | Interface Type |
| --- | --- |
| 0x80 | RS232 |
| 0x40 | RS485 |
| 0x20 | RJ45 |
| 0x10 | WiFi |
| 0x01 | USB |
| 0x02 | keyboard |
| 0x04 | CDC_COM |

  - Baudrate: Serial port baud rate, the default value is 4, the specific meanings are as follows:

| baudrate | Actual baud rate |
| --- | --- |
| 0 | 9600bps |
| 1 | 19200 bps |
| 2 | 38400 bps |
| 3 | 57600 bps |
| 4 | 115200 bps |

  - WGSet: Configuration parameters of the Wiegand data output interface, the default value is 0x00 , the specific meanings are as follows:

| WGSet | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit Definition | 0: Disable Wiegand output 1: Enable Wiegand output | 0: wg26 1: wg34 | 0: Low first 1: High position first | spare | spare | spare | Wiegand data keyboard output format |  |

- Wiegand data keyboard output format: Default is 2b 00.

- 2b 00: Convert directly to decimal characters.

- wg26 : 0xA1B2C3 is directly converted to 8-bit decimal characters as ' 10597059 ' ;

- wg 34: 0xA1B2C3D4 converts directly to 10 decimal characters as ' 2712847316 ' .

- 2b 01: Convert to decimal characters by word.

- wg26 : 0xA1B2C3 is converted to 8-bit decimal characters by 2H+4H, which is ' 16145763 ' ;

- wg 34: 0xA1B2C3D4 is converted to 10 decimal characters as ' 4139450132 ' by pressing 4H+4H .

- 2b 10: Convert directly to hexadecimal characters.

- wg26 : 0xA1B2C3 is directly expressed as 'A1B2C3' using 6-digit hexadecimal ;

- wg 34: 0x A1B2C3D4 is directly expressed as 'A1B2C3D4' using 8-bit hexadecimal .

- 2b 11: Reserved.

  - Ant: All antenna numbers of the device. The bit indicates the antenna to be used. If the corresponding bit value is 1, the antenna is used, and if the value is 0, the antenna is not used. Starting from the lowest bit, the 0th bit indicates antenna No. 1, the 1st bit indicates antenna No. 2, and so on. A maximum of 8 antennas can be represented . Different modules support different antennas, depending on the specific situation. The default value is 0x01 , indicating antenna No. 1 .

  - RfidFreq: RFID frequency-related parameters of the device, used to select the frequency band and the upper and lower frequency limits in each frequency band. The length is 8 bytes. The specific parameters are as follows:

| RfidFreq |  |  |  |  |
| --- | --- | --- | --- | --- |
| R EGION | STRATFREI | STRATFRED | STEPFRE | CN |
| 1 Byte | 2Byte | 2Byte | 2Byte | 1 Byte |

Definition of each byte:

To set the frequency, first determine the frequency band range according to REGION, and then refer to STRATFREI , STRATFRED , STEPFRE , and CN to calculate the specific frequency range.

| Fields | byte | describe |
| --- | --- | --- |
| REGION | 1 | Frequency band ranges in various countries: 0x 00: customized by the user according to needs; 0x01: US [902.75~927.25] 0x02: Korea [917.1~923.5] 0x03: EU [865.1~868.1] 0x 04 ：JAPAN [952.2~953.6] 0x05 : MALAYSIA [ 919.5 ~922.5] 0x06 : EU3 [865.7~867,5 ] 0x07 : CHINA_BAND1 [ 840.125~844.875] 0x08 : C HINA_BAND2 [ 920.125~924.875] |
| STRATFREI | 2 | in megahertz ; for example, 920.125MHz, STRATFREI = 920 = 0x 0 398, high byte = 0x03, low byte = 0x98; |
| STRATFRED | 2 | in megahertz ; for example, 920.125MHz, STRATFRED = 125, high byte = 0x00, low byte = 0x7D |
| STEPFRE | 2 | Frequency step ( KHz ) , please refer to the calculation formula of each frequency band ; for example, 125KHz, STEPFRE = 125, high byte = 0x00, low byte = 0x7D; |
| CN | 1 | Number of channels; |

Frequency calculation formula: 	Minimum frequency: Fmin = STRATFREI + STRATFRED /1000  (Unit: MHz)

Maximum frequency: Fmax = Fmin + STEPFRE *CN/1000 (Unit: MHz)

Calculation formula for each frequency band: (The frequency range must be within the standards of each country )

Chinese band2: 		Fs = 920.125 + CN * 0.25 (MHz) where CN∈[0, 19].

US band: 				Fs = 902.75 + CN * 0.5 (MHz) where CN∈[0,49].

Korean band: 			Fs = 917.1 + CN * 0.2 (MHz) where CN∈[0, 31].

EU band: 				Fs = 865.1 + CN*0.2(MHz) where CN∈[0, 14].

Ukraine band: 			Fs = 868.0 + CN*0.1(MHz) where CN∈[0, 6].

Peru band: 			Fs = 916.2 + CN *0.9(MHz) where CN∈ [0, 11].

Chinese band1: 		Fs = 840.125 + CN * 0.25 (MHz) where CN∈[0, 19].

EU3 band: 			Fs = 865.7 + CN * 0.6(MHz) where CN∈[0, 3].

US band3: 			Fs = 902 + CN * 0.5 (MHz) where CN∈[0,52].

Taiwan band: 			Fs = 922.25 + CN * 0.5 (MHz) where CN∈[0,11].

  - RfidPower: The RFID output power of the device, in dBm. The value range is [ 0 , 30]dBm. Other values are invalid .

  - InquiryArea: The storage area of the tag that the device wants to access. 0x00: Reserved area; 0x01 (default): EPC storage area ; 0x02: TID storage area; 0x03: USER storage area; 0x04: EPC+TID; 0x05: EPC+USER; 0x06: EPC+TID+USER; other values are reserved. If other values appear in the command, a parameter error message will be returned.

  - QValue: The initial Q value used when querying EPC tags. The Q value should be set to approximately equal 2Q to the number of tags in the field. The default value of Q is 4 , and the range of Q is 0 to 15. If other values appear in the command, a parameter error message will be returned.

  - Session: The Session value used when querying the EPC tag. The default value is 0 and the value range is [0, 3]. Other values will return a parameter error message.

| Session | Notes |
| --- | --- |
| 0 | Session uses S0 |
| 1 | Session uses S1 |
| 2 | Session using S2 |
| 3 | Session using S3 |
| 0xFD | Auto-3 |
| 0xFE | Auto-2 |
| 0xFF | Auto-1 |

  - AcsAddr: The starting address of the tag storage area that the device wants to access, unit: Byte, default value: 0x00: When accessing the EPC area, 0x00 indicates accessing the starting address of the EPC number segment of the EPC area except the CRC and PC segments; when accessing other storage areas, 0x00 indicates the starting address of the storage area.

  - AcsDataLen: The data length that the device wants to access the tag storage area, unit: Byte, default value: 0x00.

  - FilterTime: filter time. After a tag data is successfully read, the tags with the same data are filtered out within this time. The unit is S, and the value range is [ 0 , 255 ] . Other values are invalid . The default value is 0, which means no filtering.

  - TriggerTime: The query duration after the device receives the trigger signal, in seconds. The default value is 1. The value range is [ 0 , 255 ] . Other values are invalid .

  - BuzzerTime: The duration of the buzzer after the device is successfully executed. The unit is 10ms. The value range is [ 0 , 255 ] . Other values are invalid . The default value is 1. When it is 0, it means that the buzzer does not sound.

  - PollingInterval: polling interval, unit: 10ms, value range: [ 0 , 255 ] , other invalid , default is 1 .

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0xXX​​ | 0x00 71 | 0x01 | 1 Byte | 2Byte |

  - STATUS:

  - 0x00: command executed successfully;

  - 0x01: parameter error ;

  - Other values: invalid;

2.2.9​ RFM_GET_ ALL_PARAM ( Read all configurable parameters )

This command is used to read the module's current RF power setting. Users can use this command to view the module's power setting before using the module to operate the tag.

- Format

| HEAD | ADDR​ | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF​ | 0x00 72 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STATUS | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF​ | 0x00 71 | 0x 1 A | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes |

  - STATUS：

  - 0x00: command executed successfully;

  - Other values: invalid ;

  - For other parameters, refer to RFM_SET_ALL_PARAM.

2.2.10​ RFM_ S ET_GET_PERMISSION _PARAM (Set read permission)

This command is used to set password and mask query permission parameters, relay usage parameters, cache switch parameters and public protocol usage parameters.

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  | C HECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | 0x16 | 1 Byte | 1 Byte | 4 Bytes​ | 1 Byte | 1 Byte | 1 Byte | 12 Bytes​ | 1 Byte | 2 Bytes​ |

Parameter explanation:

Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

CodeEN: Password function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

Codes: Access code of the tag, 4 bytes in length. Default value is [0x00,0x00,0x00,0x00] .

MaskEN: Mask function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

StartAdd: mask start address, length 1 byte, unit byte. Default 0x00 .

Mask Len: Mask length, 1 byte, unit byte, maximum 12. Default 0x00 .

MaskData: mask data, 12 bytes in length. When the mask length is less than 12 bytes, the following bytes are padded with 0.

Default is [0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00] .

MaskCondition: Mask condition, length 1 byte . 0x00: password or mask meets the requirement; 0x01: password and mask meet the requirement at the same time. Default value is 0x00 .

- Response format and status byte

- Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x00 73 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

- Get the response:

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | 0x17 | 1Byte | 1Byte | 1Byte | 4 Byte | 1 Byte | 1 Byte | 1 Byte | 12 Bytes​ | 1 Byte | 2 Bytes​ |

  - STATUS:

  - 0x00: command executed successfully;

  - Other values: invalid ;

2.2.11​​​ RFM_SET_ G ET_IOPUT _PARAM (Set read input and output control)

This command is used to read the advanced parameters of the device. For detailed analysis of the parameters, please refer to 2.2.9.

- Command Format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Relay EN | Relay Time | Relay Power EN | Trig Way | Buffer EN | Protocol EN | Protocol Type | Protocol Format |  |
| 0xCF | 0xFF | 0x007 4 | 0x12 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 10 Bytes​ | 2 Bytes​ |

Parameter explanation:

Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

RelayEN: relay function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default is 0x00 .

RelayTime: relay normally open foot conduction time parameter, length 1 byte, unit S. Default 0x03 .

Relay PowerEN: relay common terminal power on enable parameter, length 1 byte . 0x01 is on , 0x00 is off . Default is 0x00 .

TrigWay: Valid trigger mode parameter of the trigger pin, 1 byte in length . 0x01 high level (2V-5V) trigger, 0x00 low level (less than 0.5V) trigger. Default 0x01 .

BufferEN: Buffer function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

ProtocolEN: Specific data exchange function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

ProtocolType: Specific data exchange protocol type, length 1 byte. Default 0x00 .

Modbus-RTU:0x00; ASCII:0x01; HEX:0x02; Modbus-TCP:0x03;

ASCII_Frame: 0x04, Text: 0x02+ASCII+0x03;

ASCII_Enter_Frame:0x05, text: 0x02 + ASCII + 0x0D + 0x0A + 0x03;

ASCII_CVS_Enter_Frame: 0x06, text when the read/write area is EPCandTIDandUSER:

0x02 + EPC (ASCII string) + ',' + TID (ASCII string) + ',' + USER (ASCII string) + ',' +

Ant (see 2.3.1, decimal string) + ',' + RSSI (see 2.3.1, signed decimal string) +

0x0D + 0x0A + 0x03;

ProtocolFormat: Specific data exchange protocol format, length 10 bytes .

Default is [0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00] .

- Response format and status byte

- Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x00 73 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

- Get the response:

| HEAD | Control Field |  |  |  | PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN |  | STATUS | OPTION | Relay EN | Relay Time | Relay Power EN | Trig Way | Buffer EN | Protocol EN | Protocol Type | Protocol Format |  |
| 0xCF | 0xFF | 0x0074 | 0x13 |  | 1Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 10 Bytes​ | 2 Bytes​ |

  - STATUS:

  - 0x00: command executed successfully;

  - Other values: invalid ;

2.2.12 RFM_SET_GET_WiFi_PARAM (Set/Read WiFi Information)

This command sets/reads the module's WiFi information. If it is set as TCP Client, the corresponding remote IP address and port need to be set.

Wi-Fi network configuration mode: 1. SmartConfig configures the network through the mobile phone app, 2. Serial port configuration, configures by sending ssid and pwd. After successful configuration, it will automatically connect after power on.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | WiFiPara |  |
| 0xCF | 0xFF | 0x0075 | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Setting;

0x02: read;

Other values: invalid;

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0075 | 1Byte | 1Byte | 1Byte | 2Byte |

获取响应：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x00 | 0x0075 | 1Byte | 1Byte | 1 Byte | NByte | 2Byte |

| Fields |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option |  | 1 | 0x01: Set; 0x02: Get |
| WiFiPara | WiFiEN | 1 | WiFi enabling parameters , mode selection . bit0: 0: turn off wifi; 1: turn on wifi bit1: 0: smartconfig 1: serial port network configuration. Other bits default to 0; |
|  | SSID | 32 | Target WiFi name, Asic code. Invalid bytes are filled with 0x00 |
|  | PWD | 64 | Target WiFi password, ASIC code. Invalid bytes are filled with 0x00 |
|  | RemoteIPAddr | 4 | Remote IP address. If the IP address is 192.168.1.1, the data is [0xC0, A8, 0x01, 0x01] |
|  | RemotePort | 2 | Remote IP port number, high byte first, low byte last |

2.2.13​ RFM_ S ET_GET_S-PERMISSION _PARAM (Set read permission)

This command is used to set password and mask query permission parameters, relay usage parameters, cache switch parameters and public protocol usage parameters.

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0076 | 0x29 | 1Byte | 1Byte | 4Bytes | 1Byte | 1 Byte | 1 Byte | 31Bytes​ | 1 Byte | 2 Bytes​ |

Parameter explanation:

Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

CodeEN: Password function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

Codes: Access code of the tag, 4 bytes in length. Default value is [0x00,0x00,0x00,0x00] .

MaskEN: Mask function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

StartAdd: mask start address, length 1 byte, unit byte. Default 0x00 .

Mask Len: Mask length, 1 byte, unit byte, maximum 31. Default 0x00 .

MaskData: mask data, 31 bytes in length. When the mask length is less than 31 bytes, the following bytes are padded with 0.

The default is all 0 .

MaskCondition: Mask condition, length 1 byte . 0x00: password or mask meets the requirement; 0x01: password and mask meet the requirement at the same time. Default value is 0x00 .

- Response format and status byte

- Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0073 | 1Byte | 1Byte | 1Byte | 2Byte |

- 获取响应：

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x007 6 | 0x 29 | 1 Byte | 1 Byte | 0xFF | 4 0xFF | 1 Byte | 1 Byte | 1 Byte | 31Bytes​ | 1 Byte | 2 Bytes​ |

  - STATUS:

- 1) 0x00: command executed successfully;

- Other values: invalid ;

2.2.1 4 RFM_RELEASE_CLOSE_RELAY1 (Release/Close Relay 1)

This command is used to release and close the relay.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | Valid'Time |  |
| 0xCF | 0xFF | 0x007 7 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

  - Option: command control option

0x01: Release ;

0x02: Closed ;

- Other values: invalid;

  - ValidTime : Valid time when closed, unit is S, the default value is 1. When the ValidTime value is equal to 0, relay 1 will remain in the closed state; when the ValidTime value is greater than 0, relay 1 will be released after closing for ValidTime seconds.

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | Option | Valid'Time | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x007 7 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Byte |

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option | 1 | 0x01: Release ; 0x02: Close |
| ValidTime | 1 | The effective time when closed, in seconds, the default value is 1. |

2.2.1 5 RFM_RELEASE_CLOSE_RELAY2 (Release/Close Relay 2)

This command is used to release and close the relay.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | Valid'Time |  |
| 0xCF | 0xFF | 0x007 8 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

  - Option: command control option

0x01: Release ;

0x02: Closed ;

- Other values: invalid;

  - ValidTime : Valid time when closed, unit is S, the default value is 1. When the ValidTime value is equal to 0, relay 2 will remain in the closed state; when the ValidTime value is greater than 0, relay 2 will be released after closing for ValidTime seconds.

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | Option | Valid'Time | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x007 8 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Byte |

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option | 1 | 0x01: Release ; 0x02: Close |
| ValidTime | 1 | The effective time when closed, in seconds, the default value is 1. |

2.2.16​ RFM_ S ET_GET_AntN-RSSI-Filter (set the RSSI filter threshold for reading each antenna port)

This command is used to filter tags with different signal strengths . RSSI is in negative form, and the larger the absolute value, the farther the tag is.

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | C HECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | OPTION | BasicRSSI | AntN-RSSIData |  |
| 0xCF | 0xFF | 0x007 9 | 0x13 | 1 Byte | 2Byte | Ant1-RSSIData~Ant16-RSSIData 16Byte | 2 Bytes​ |

Parameter explanation:

Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

BasicRSSI: The basic value of the RSSI filter threshold of each antenna port, 2 bytes . The value range is [-32767, 32767]. Negative numbers are in the complement format. The default value is 0xFFBF: -65.

AntN-RSSIData : RSSI filter threshold offset value of each antenna port , 16 bytes . The RSSI filter threshold offset value of each antenna port occupies 1 byte, with a value range of [0, 255] and a default value of 0.

Note: The RSSI filter value for each antenna port X is BasicRSSI - AntX-RSSIData. For example: BasicRSSI is -65, Ant1-RSSIData is 1, then the RSSI filter value for antenna 1 is -66. When the RSSI filter value is less than -174, such as -175, the filter function will not work.

- Response format and status byte

- Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0079 | 0x02 | 1Byte | 1Byte | 2Byte |

- 获取响应：

| HEAD | Control Field |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | BasicRSSI | AntN-RSSIData |  |
| 0xCF | 0xFF | 0x0079 | 0x 14 | 1 Byte | 1 Byte | 2Byte | Ant1-RSSIData~Ant16-RSSIData 16Byte | 2 Bytes​ |

  - STATUS:

- 1) 0x00: command executed successfully;

- 2) Other values: invalid ;

2.2.17​​ RFM_TEST_LEDCFG ( Set LED Status )

This command is used to configure the status of the LED external light and command control;

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | LED NO. | LED Color |  |
| 0xCF | 0xFF | 0x00E9 | 0x 03 | 1 Byte | 1 Byte | 1 Byte | 2 Bytes​ |

Parameter explanation:

Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

LED NO.: The serial number of the external light strip, starting from 0 by default;

LED Color : The color of the LED light is controlled by bit: the default is 0x00; 1 means it is on;

Bit0: Red

Bit1: Green

Bit2: Blue

For example, if a white light is needed, it is 0x07;

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x00E9 | 0x02 | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | Control Field |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | LED NO. | LED Color |  |
| 0xCF | 0xFF | 0x00E9 | 0x04 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2 Bytes​ |

  - STATUS:

1) 0x00: command executed successfully;

2) Other values: invalid;

2.3 International standard (ISO 18000-6 C ) protocol related commands

2.3.1 RFM_INVENTORYISO_CONTINUE ( Label Inventory )

This command starts the international standard multi-tag inventory command, and the inventory function has an anti-collision algorithm.

This command can specify the duration of the inventory count. If the input inventory count count is 0, it means that the inventory will continue until the stop inventory count command is received. During the inventory count process, each time a tag is successfully counted, the command response with a "STATUS" value of "0" will return the information of the newly counted tag.

When the inventory command is successfully executed, the command response with a "STATUS" value of "0x12" will be returned to notify that the inventory command has been executed.

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | InvType | InvParam |  |
| 0xCF | 0xFF​ | 0x0001 | 0x05 | 1 Byte | 4Byte | 2Byte |

  - InvType: Inventory method:

0x00: count labels by time, stop counting after executing the specified time or after receiving the stop counting command;

0x 01: Inventory is performed according to the number of cycles. The inventory is stopped after the specified number of polls is executed or after receiving the stop inventory command;

  - InvParam: Inventory method parameters:

1. If InvType is 0x0 0 :

InvParam indicates the inventory time in seconds. If the value is 0, it means that the inventory will continue until a stop command is received.

2. If InvType is 0x0 1 :

InvParam indicates the number of inventory counts , in times , and the value must be greater than 0;

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | RSSI | Antenna | Channel | EPC LEN | EPC NUM |  |
| 0xCF | 0x00​ | 0x0001 | 1 Byte | 1 Byte | 2Bytes | 1 Byte | 1 Byte | 1 Byte | N Bytes | 2Byte |

Definition of each byte:

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: The tag is successfully counted and the tag information is included in PAYLOAD; 0x01: Q, MemBank parameter value is incorrect or Length is inconsistent with Mask data length; 0x02: Command execution failed due to an internal error in the module; 0x12: No tag is counted or the entire count command is executed; 0x17: The tag data exceeds the maximum transmission length of the serial port; Other values: invalid; |
| RSSI | 2 | RSSI of tag ACK response, in dBm, signed number, negative number uses two's complement format; |
| Antenna | 1 | Antenna port number |
| Channel | 1 | The channel from which the tag data is received. The value starts from 0, where 0 represents channel 0, 1 represents channel 1, and so on. |
| EPC LEN | 1 | The length of the tag's EPC number (bytes) ; |
| EPC NUM | N Byte | The EPC number of the label ; |

2.3.2 RFM_INVENTORY_STOP ( Stop Inventory )

This command allows the user to actively stop the international standard multi-tag anti-collision inventory process.

- Format

| HEAD | ADDR​ | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF​ | 0x00 02 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 02 | 0x01 | 1 Byte | 2Byte |

 STATUS: This command will only return a successful execution (value 0x00), other values are invalid;

2.3.3 RFM_READISO_TAG ( Read tag data )

This command frame is for the module to read the data in the international standard protocol tag storage area.

Before calling this method, please set the inventory condition (for selecting tags) through the RFM_SETISO_SELECTMASK command. When performing international standard protocol tag data operations (reading tag data, writing tag data, locking tags, and deactivating tags), the tag will return the tag operation status. At this time, the module's response contains a byte of tag operation status. The definition of tag operation status is shown in Appendix A.

- Format

| HEAD | ADDR​ | CMD | LEN |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | AccPwd | MemBank | WordPtr | WordCount |  |
| 0xCF | 0xFF​ | 0x00 03 | 1 Byte | 1 Byte | 4 Bytes | 1 Byte | 2 Bytes | 1 Bytes | 2Byte |

Definition of each field:

| Fields | byte | describe |
| --- | --- | --- |
| Option | 1 | Command options (this field is not used, invalid , default value is 0x00 ) ; |
| AccPwd | 4 | Access password, used for the tag to enter the secure state, the default value is 0x00000000; |
| MemBank | 1 | The storage area of the tag to be read, the value list is as follows : 0x00: Reserved; 0x01: EPC; 0x02: TID; 0x03: User; |
| WordPtr | 2 | (word ) of the logical storage area ; |
| WordCount | 1 | The number of words to be read ; - When WordCount is equal to 0 and MemBank is not 01, the data content returned by the tag is from the starting address defined by WordPtr to the end of the storage area. - When WordCount is equal to 0 and MemBank is equal to 01, the data content returned by the tag is from the starting address defined by WordPtr to the EPC length specified by the first 5 bits of PC. |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM | WordCount | Data |  |
| 0xCF | 0x00 | 0x0003 | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte | 2Bytes | 1Byte | N Bytes | 1Bytes | N-Byte | 2Byte |

各字段定义：

based on STATUS and TagStatus .

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | From which antenna is the reading |
| CRC | 2 | CRC data in the tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes) ; |
| EPC NUM | N Byte | The EPC number of the label ; |
| WordCount | 1 | The number of tag data words successfully read ; |
| Data | N Byte | The tag data successfully read has a length of WordCount×2 bytes; |

2.3.4​ RFM_WRITEISO_TAG ( write tag data )

This command is used by the module to write data into the tag storage area.

Before calling this method, please set the inventory conditions (for selecting tags) by using the RFM_SETISO_SELECTMASK command.

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | AccPwd | MemBank | WordPtr | WordCount | Data |  |
| 0xCF | 0xFF​ | 0x00 04 | 1 Byte | 1 Byte | 4Byte | 1 Byte | 2Byte | 1 Byte | N-Byte | 2Byte |

Definition of each field:

| Fields | byte | describe |
| --- | --- | --- |
| Option | 1 | Command options (this field is not used, invalid , default value is 0x00 ); |
| AccPwd | 4 | Access password, used for the tag to enter the secure state, the default value is 0x00000000; |
| MemBank | 1 | The storage area of the tag to be read, the value list is as follows : 0x00: Reserved; 0x01: EPC; 0x02: TID; 0x03: User; |
| WordPtr | 2 | (word) of the logical storage area ; |
| WordCount | 1 | The number of data words that need to be written into the tag (1 word is two bytes), must be greater than 0; |
| Data | N-Byte | The data to be written to the tag must be an integer multiple of words, with a length of 1 to WordCount words. |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM |  |
| 0xCF | 0x00​ | 0x00 04 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Byte | 2Bytes | 1 Byte | N Bytes | 2Byte |

  - TagStatus: The operation status returned by the tag. For specific definitions, see Appendix A. This field is included in the response data only when the value of the STATUS field is 0.

Definition of each field:

based on STATUS and TagStatus .

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | From which antenna port the tag data is received. The value range is: 1~4, representing antennas 1~4 respectively. |
| CRC | 2 | CRC data in the tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes) ; |
| EPC NUM | N Byte | The EPC number of the label ; |

2.3.5 RFM_LOCKISO_TAG ( Lock tag data )

This command locks the specified data storage area of the specified international standard protocol tag.

Before calling this method, please set the inventory conditions (for selecting tags) by using the RFM_SETISO_SELECTMASK command.

- Format

| HEAD | ADDR | CMD | LEN |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AccPwd | Area | Action |  |
| 0x CF | 0xFF | 0x00 05 | 0x06 | 4Byte | 1 Byte | 1 Byte | 1 Byte |

Definition of each field:

| Fields | byte | describe |
| --- | --- | --- |
| AccPwd | 4 | Access password, used for the tag to enter the safe state, the default value is 0x00000000 |
| Area | 1 | The area that needs to be locked, the value list is as follows : 0x00: Kill password area ; 0x01 : Access password area; 0x02 : EPC; 0x03 : TID; 0x04 : User; |
| Action | 1 | Lock operation type, the value list is as follows : 0x00: open ; 0x01: permanently open ; 0x02: locked ; 0x03: permanently locked ; |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM |  |
| 0x CF | 0x00 | 0x00 05 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Bytes | 2Bytes | 1 Byte | N Bytes | 1 Byte |

Definition of each field:

based on STATUS and TagStatus .

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | The antenna port from which the tag data is received. The value range is: 1~4, representing antennas 1 to 4 respectively. |
| CRC | 2 | CRC data in the tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes) ; |
| EPC NUM | N Byte | The EPC number of the label ; |

2.3.6 RFM_KLENLISO_TAG ( Kill tag )

This command is used to deactivate the module tag.

Before calling this method, please set the inventory conditions (for selecting tags) by using the RFM_SETISO_SELECTMASK command.

Kill, only supports non-zero kill password.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | KLnP |  |
| 0x CF | 0xFF | 0x00 06 | 0x04 | 4Byte | 1 Byte |

  - KLENlPwd: KLENl command password;

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM |  |
| 0x CF | 0x 00 | 0x00 06 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Byte | 2Byte | 1 Byte | N Bytes | 1 Byte |

Definition of each field:

based on STATUS and TagStatus .

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | The antenna port from which the tag data is received. The value range is: 1~4, representing antennas 1 to 4 respectively. |
| CRC | 2 | CRC data in the tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes) ; |
| EPC NUM | N Byte | The EPC number of the label ; |

2.3.7​ RFM_SETISO_SELECTMASK ( Select the tag to be operated )

This command is used to set the tag matching information required for tag operations (read, write, lock and kill).

- Format

| HEAD | ADDR​ | CMD | LEN |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pointer | Length | Mask |  |
| 0xCF | 0xFF​ | 0x00 07 | 1 Byte | 2Bytes | 1 Byte | N-Bytes | 2Byte |

  - Pointer: reserved, the default value is 0x0000;

  - The bit length of the EPC number to be matched . The default value is 0x00. If Length is 0, it means that no tag is specified and multiple tags are operated.

  - Mask: data to be matched. The effective data length is Length bits. If Length is an odd number, the lower bits of the mask need to be padded with 0.

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 07 | 0x01 | 1 Byte | 2Byte |

  - STATUS:

  - 0x00: command executed successfully;

  - 0x01: parameter error;

  - Other values: invalid;

2.3.8​ RFM_SET_SELPRM ( Set Select command parameters )

This command is used to set the Select instruction parameters of the module's specified protocol, including "MemBank", "Target", "Action", "Pointer", "Length", and "Mask". For the specific application and meaning of each parameter, please refer to the protocol description.

- Format

| HEAD | CMD | LEN |  |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protocol | Target | Action | Membank | Pointer | Length | Mask |  |
| 0xCF | 0x00 10 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Bytes | 1 Byte | N-Bytes | 2 Byte |

  - Protocol: Protocol type

0x00 : ISO

  - Target: Select to modify the target.

0x00 : S0 ; 0x01 : S1 ; 0x02 : S2 ; 0x03 : S3 ; 0x04 : SL ; 0x05-0xFF : reserved .

  - Action: Select the matching condition rule

| Action | ISO |
| --- | --- |
| 0x00 | If the conditions are met, set it to SL or A; if the conditions are not met, set it to non-SL or B |
| 0x01 | Eligible conditions are set to SL or A |
| 0x02 | If the conditions are not met, set to non-SL or B |
| 0x03 | Meet the conditions to reverse SL or A->B, B->A |
| 0x04 | If the conditions are met, it is set to non-SL or B; if the conditions are not met, it is set to SL or A |
| 0x05 | Eligible conditions are set to non-SL or B |
| 0x06 | If the conditions are not met, set to SL or A |
| 0x07 | Does not meet the conditions to reverse SL or A->B, B->A |

  - MemBank: The storage area where the selected and matched data is located

| Membank | ISO | Interpretation | Remark |
| --- | --- | --- | --- |
| 0x00 | RFU | Label information area | Label information area |
| 0x10 | EPC | Coding region | Coding region |
| 0x20 | TID | Safe Zone | Safe Zone |
| 0x30 | User | User Area | User subarea 0 |

  - Pointer: The starting address of the match, in bits

  - Length: The length of the matching mask, in bits

  - Mask: data to be matched. The effective data length is Length bits. If Length is an odd number, the lower bits of the mask need to be padded with 0.

- Response format and status byte

| HEAD | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocol |  |
| 0xCF | 0x00 10 | 1 Byte | 1 Byte | 1 Byte | 2 Byte |

  - Status: command response code

0x00: command executed successfully;

0x01: parameter error;

Other values: invalid;

  - Protocol: Protocol type

0x00 : ISO

2.3.9​ RFM_GET_SELPRM ( Get Select command parameters )

This command is used to obtain the Select/Sort instruction parameters of the specified protocol of the module, including "MemBank", "Target", "Action", "Pointer", "Length", and "Mask". For the specific application and meaning of each parameter, please refer to the protocol description.

- Format

| HEAD | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- |
|  |  |  | Protocol |  |
| 0xCF | 0x00 11 | 1 Byte | 1 Byte | 2 Byte |

  - Protocol: Protocol type

0x00 : ISO

- Response format and status byte

| HEAD | CMD | LEN |  |  |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocol | Target | Action | Membank | Pointer | Length | Mask |  |
| 0xCF | 0x00 11 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Bytes | 1 Byte | N-Bytes | 2 Byte |

  - Status: command response code

0x00: command execution successful; 0x01: parameter error; other values: invalid;

  - Protocol: Protocol type

0x00 : ISO

  - Target: Select to modify the target.

0x00 : S0 0x01 : S1 0x02 : S2 0x03 : S3 0x04 : SL 0x05-0xFF : Reserved

  - Action: Select the matching condition rule

| Action | ISO |
| --- | --- |
| 0x00 | If the conditions are met, set it to SL or A; if the conditions are not met, set it to non-SL or B |
| 0x01 | Eligible conditions are set to SL or A |
| 0x02 | If the conditions are not met, set to non-SL or B |
| 0x03 | Meet the conditions to reverse SL or A->B, B->A |
| 0x04 | If the conditions are met, it is set to non-SL or B; if the conditions are not met, it is set to SL or A |
| 0x05 | Eligible conditions are set to non-SL or B |
| 0x06 | If the conditions are not met, set to SL or A |
| 0x07 | Does not meet the conditions to reverse SL or A->B, B->A |

  - MemBank: The storage area where the selected and matched data is located

| Membank | ISO | Interpretation | Remark |
| --- | --- | --- | --- |
| 0x00 | RFU | Label information area | Label information area |
| 0x10 | EPC | Coding region | Coding region |
| 0x20 | TID | Safe Zone | Safe Zone |
| 0x30 | User | User Area | User subarea 0 |

  - Pointer: The starting address of the match, in bits

  - Length: The length of the matching mask, in bits

  - Mask: data to be matched. The effective data length is Length bits. If Length is an odd number, the lower bits of the mask need to be padded with 0.

2.3.10​ RFM_GET_QUERY_PARAM ( Get Query command parameters )

This command is used to obtain the Query instruction parameters of the specified protocol of the module, including "Sel", "Session", and "Target". For the specific application and meaning of each parameter, please refer to the protocol description .

- Format

| HEAD | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- |
|  |  |  | Protocol |  |
| 0xCF | 0x00 13 | 1 Byte | 1 Byte | 2 Byte |

  - Protocol: Protocol type

0x00 : ISO

- Response format and status byte

| HEAD | CMD | LEN |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocol | Sel | Session | Target |  |
| 0xCF | 0x00 13 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2 Byte |

  - Status: command response code

0x00: command execution successful; 0x01: parameter error; other values: invalid;

  - Protocol: Protocol type

0x00 : ISO

  - Sel: Specifies the label of the inventory response

0x00 : ALL; 0x01:ALL; 0x02: ~SL; 0x03: SL .

  - Session: Specifies the section of the inventory label. This field is valid for ISO and national standards, and is not used for national standards.

0x00: S0 ; 0x01: S1 ; 0x02: S2 ; 0x03: S3

  - Target : Inventory identification

0x00: A

0x01: B

2.3.11​ RFM_SET_QUERY_PARAM ( Set Query command parameters )

This command is used to set the Query instruction parameters of the specified protocol of the module, including "Sel", "Session", and "Target". For the specific application and meaning of each parameter, please refer to the protocol description.

- Format

| HEAD | CMD | LEN |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protocol | Sel | Session | Target |  |
| 0xCF | 0x00 12 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2 Byte |

  - Protocol: Protocol type

0x00 : ISO

  - Sel: Specifies the label of the inventory response

0x00 : ALL; 0x01:ALL; 0x02: ~SL; 0x03: SL .

  - Session: Specifies the section of the inventory tag

0x00: S0 ; 0x01: S1 ; 0x02: S2 ; 0x03: S3

  - Target : Inventory identification

0x00: A

0x01: B

- Response format and status byte

| HEAD | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocol |  |
| 0xCF | 0x00 12 | 0x02 | 1 Byte | 1 Byte | 2 Byte |

  - Status: command response code

0x00: command execution successful; 0x01: parameter error; other values: invalid;

  - Protocol: Protocol type

0x00 : ISO

2.4 MQTT related commands

2.4.1 JSON message body

Subscription and publication message body:

{ "id": "iUA4fO7yCGrHZO2C", # Message ID, used to associate specific messages "bv": 12378, # Development board version number "sn": "265e1040-85e01fb7", # Device serial number "name": "ivs_result", # message name "version": "1.0", # Message version, currently set to 1.0 "timestamp": 1621926595, # timestamp "payload": {} #message body }

Receipt message body:

{ "id": "NYGtiXpPy5ratyzU", # Message ID, used to associate specific messages "sn": "12345678-87654321", # Device serial number "name": "ivs_trigger", # message name "code": 200, # Whether the device successfully processed the request, 200 for success, non-200 for failure "version": "1.0", # Message version, currently set to 1.0 "timestamp": 1597285865, # timestamp "payload": null # message data, depends on the specific message }

Heartbeat packet message body:

{ "id": "iUA4fO7yCGrHZO2C", # Message ID, used to associate specific messages "bv": 12378, # Development board version number "sn": "265e1040-85e01fb7", # Device serial number "name": "/heartbeat", # message name "version": "1.0", # Message version, currently set to 1.0 "timestamp": 1621926595, # timestamp "payload": { #message body "type": "heartbeat", "body": { "timestamp": 1621926595 } } }

2.4.2 RFM_SET_GET_MQTT_ConnectInfo (Set/Read MQTT connection information)

- Format

| HEAD | Control Field |  |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | MQTT Connect Info |  |
| 0xCF | 0xFF | 0x007B | 233 , 1 Byte | 1 Byte | 232 Bytes | 2 Bytes​ |

- Response Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | Status | OPTION | MQTT Connect Info |  |
| 0xCF | 0xFF | 0x007B | 234 , 1 Byte | 1 Byte | 1 Byte | 232 Bytes | 2 Bytes​ |

Parameter explanation:

Status : Command execution result status word, 1 Byte, can only be 0x00: success, other values are invalid.

Option : command control option, 0x01 is set, in the command format, it is followed by MQTT Connect Info , in the response format, it is not connected; 0x02 is read, in the response format, it is followed by MQTT Connect Info , in the command format, it is not connected; other values are invalid.

MQTT Connect Info : MQTT connection information, 232 Bytes, the specific composition is as follows:

| MSB LSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MQTT Enable | MQTT Server IP Add | MQTT Server IP Port | Keep Alive Max Inv Time | MQTT Protocol | Client ID | Username | Password | Publish Heartbeat Max Inv Time | Publish Heartbeat Topic |
| 1 Byte | 4Bytes | 2Bytes | 2Bytes | 1 Byte | 25 Bytes | 65 Bytes | 65 Bytes | 2Bytes | 65 Bytes |

MQTT Enable : Enable MQTT function, 1Byte , default 0. 0 : Disable MQTT function; 1 : Enable MQTT function.

MQTT Server IP Add : MQTT server IP address, 4 Bytes , default 0x2B8B42C9 , corresponding IP address is 43.139.66.201 .

MQTT Server IP Port : MQTT server IP port number, 2 Bytes , default 0x075B, corresponding IP port number is 1883.

Keep Alive Max Inv Time : The maximum interval time to maintain the connection, 2 Bytes , unit is seconds, the default value is 10 .

MQTT Protocol : MQTT protocol, 1 Byte . 0x00 : MQTT V3.1 ; 0x01 : MQTT V3.1.1 ; other values are invalid.

Client ID : Client identifier, consisting of Client ID Len ( N0 , 2 Bytes ) plus Client ID Data ( 23 Bytes ), N0 = [1,23] . The first N0 bytes of Client ID Data are Client ID , and the remaining bytes are padded with 0 .

Username : Username, which is composed of Username Len ( N1 , 2 Bytes ) plus Username Data ( 63 Bytes ), N1=[0,63] . The first N1 bytes of Username Data are Username , and the remaining bytes are filled with 0 .

Password : Password, composed of Password Len ( N2 , 2 Bytes ) plus Password Data ( 63 Bytes ), N2=[0,63] .

N2 bytes in Password Data are Password , and the remaining bytes are filled with 0 .

Publish Heartbeat Max Inv Time : The maximum interval for publishing heartbeat packets, 2 Bytes , in seconds; the default value is 30. When it is 0 , the heartbeat packet topic is not published.

Publish Heartbeat Topic : Publish heartbeat packet topic, which consists of Publish Heartbeat Topic Name Len ( N3 , 2 Bytes ) plus Publish Heartbeat Topic Name Data ( 63 Bytes ), N3=[0,63] . The first N3 bytes in Publish Heartbeat Topic Name Data are Publish Heartbeat Topic Name , and the remaining bytes are filled with 0 .

2.4.3 RFM_SET_GET_Subscribe_SerialData_TopicInfo (Set/Read serial data topic information)

- Format

| HEAD | Control Field |  |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Topic1 Info |  |
| 0xCF | 0xFF | 0x007C | 133，1Byte | 1Bytes | 132 Bytes | 2Bytes |

- Response Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | Status | OPTION | Topic1 Info |  |
| 0xCF | 0xFF | 0x007C | 134 ， 1 Byte | 1 Byte | 1 Byte | 132 Bytes​ | 2 Bytes​ |

Parameter explanation:

Status : Command execution result status word, 1 Byte, can only be 0x00: success, other values are invalid.

Option : command control option, 0x01 is set, in the command format, it is followed by Topic1 Info , in the response format, it is not connected; 0x02 is read, in the response format, it is followed by Topic1 Info , in the command format, it is not connected; other values are invalid.

Topic 1 Info : Topic 1 information, 132 Bytes , the specific composition is as follows:

| MSB LSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Subscribe Topic1 QoS | Subscribe Topic1 Name Len | Subscribe Topic1 Name Data | Publish Topic1 Receipt QoS | Publish Topic1 Receipt Name Len | Publish Topic1 Receipt Name Data |
| 1 Byte | N4, 2 Bytes | 63 Bytes​ | 1 Byte | N5, 2 Bytes | 63 Bytes​ |

Subscribe Topic1 QoS : Session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Subscribe Topic1 Name Len : Length of the name of subscription topic 1 , N4=[0,63] , 2 Bytes; when N4 = 0 , topic 1 is not subscribed .

Subscribe Topic1 Name Data : Subscribe Topic 1 name data, 63 Bytes . The first N4 bytes of Subscribe Topic1 Name Data are Subscribe Topic1 Name , and the remaining bytes are filled with 0 .

Publish Topic1 Receipt QoS : Publish topic 1 receipt session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Publish Topic1 Receipt Name Len : Length of the receipt name for Topic 1, N5=[0,63], 2 Bytes; when N5 = 0 , the receipt for Topic 1 is not published .

Publish Topic1 Receipt Name Data : Publish Topic 1 Receipt Name Data, 63 Bytes . The first N5 bytes in Publish Topic1 Receipt Name Data are Publish Topic1 Name , and the remaining bytes are filled with 0 .

Message body example:

{ "id": "CP202910MxMQTTtest", "bv": "CP202910_V1.51", "sn": "\ufffd\ufffd4633\u0011", "name": " sub_serial_data ", "version": "1.0", "timestamp": 1328075, "payload": { "type": "serial_data", "body": { "serial_data": { "serialChannel": 0, "data": "MTIzNDU2", "dataLen": 6 } } } }

2.4.4 RFM_SET_GET_Publish_SerialData_TopicInfo (Set/Read Published Serial Data Topic Information)

- Format

| HEAD | Control Field |  |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Topic2 Info |  |
| 0xCF | 0xFF | 0x007D | 133，1Byte | 1Byte | 132 Bytes | 2Bytes |

- Response Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | Status | OPTION | Topic2 Info |  |
| 0xCF | 0xFF | 0x007D | 134 ， 1 Byte | 1 Byte | 1 Byte | 132 Bytes​ | 2 Bytes​ |

Parameter explanation:

Status : Command execution result status word, 1 Byte, can only be 0x00: success, other values are invalid.

Option : command control option, 0x01 is set, in the command format, it is followed by Topic2 Info , in the response format, it is not connected; 0x02 is read, in the response format, it is followed by Topic2 Info , in the command format, it is not connected; other values are invalid.

Topic2 Info : Topic 2 information, 132 Bytes , the specific composition is as follows:

| MSB LSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Publish Topic2 QoS | Publish Topic2 Name Len | Publish Topic2 Name Data | Subscribe Topic2 Receipt QoS | Subscribe Topic2 Receipt Name Len | Subscribe Topic2 Receipt Name Data |
| 1 Byte | N5, 2 Bytes | 63 Bytes​ | 1 Byte | N6, 2 Bytes | 63 Bytes​ |

Publish Topic2 QoS : Publish topic 2 session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Publish Topic2 Name Len : Length of the name of topic 2, N6=[0,63], 2 Bytes; when N6 = 0 , topic 2 is not published .

Publish Topic2 Name Data : Publish Topic 2 name data, 63 Bytes . The first N6 bytes of Publish Topic2 Name Data are Publish Topic2 Name , and the remaining bytes are filled with 0 .

Subscribe Topic2 Receipt QoS : Subscribe to Topic 2 receipt session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Subscribe Topic2 Receipt Name Len : Length of the receipt name for subscription topic 2 , N7=[0,63] , 2 Bytes ; when N7=0 , the receipt for subscription topic 2 is not subscribed.

Subscribe Topic2 Receipt Name Data : Subscribe to Topic 2 Receipt Name Data, 63 Bytes . The first N7 bytes in Subscribe Topic2 Receipt Name Data are Subscribe Topic2 Receipt Name , and the remaining bytes are filled with 0 .

Message body example:

{ "id": "CP202910MxMQTTtest", "bv": "CP202910_V1.51", "sn": "\ufffd\ufffd4633\u0011", "name": "pub_serial_data", "version": "1.0", "timestamp": 1328075, "payload": { "type": "serial_data", "body": { "serial_data": { "serialChannel": 0, "data": "MTIzNDU2", "dataLen": 6 } } } }

2.4.5 RFM_SET_GET_Publish_TagData_TopicInfo (Set/Read Publish Tag Data Topic Information)

- Format

| HEAD | Control Field |  |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Topic3 Info |  |
| 0xCF | 0xFF | 0x007E | 133，1Byte | 1Byte | 132 Bytes | 2Bytes |

- Response Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | Status | OPTION | Topic3 Info |  |
| 0xCF | 0xFF | 0x007E | 134 ， 1 Byte | 1 Byte | 1 Byte | 132 Bytes​ | 2 Bytes​ |

Parameter explanation:

Status : Command execution result status word, 1 Byte, can only be 0x00: success, other values are invalid.

Option : command control option, 0x01 is set, in the command format, it is followed by Topic3 Info , in the response format, it is not connected; 0x02 is read, in the response format, it is followed by Topic3 Info , in the command format, it is not connected; other values are invalid.

Topic3 Info : Topic 3 information, 132 Bytes , the specific composition is as follows:

| MSB LSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Publish Topic2 QoS | Publish Topic2 Name Len | Publish Topic2 Name Data | Subscribe Topic2 Receipt QoS | Subscribe Topic2 Receipt Name Len | Subscribe Topic2 Receipt Name Data |
| 1 Byte | N8, 2 Bytes | 63 Bytes​ | 1 Byte | N9, 2 Bytes | 63 Bytes​ |

Publish Topic3 QoS : Publish topic 3 session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Publish Topic3 Name Len : Length of the name of topic 3 , N8=[0,63] , 2 Bytes; when N8 = 0 , topic 3 is not published .

Publish Topic3 Name Data : Publish Topic 3 name data, 63 Bytes . The first N8 bytes in Publish Topic3 Name Data are Publish Topic3 Name , and the remaining bytes are filled with 0 .

Subscribe Topic3 Receipt QoS : Subscribe to topic 3 receipt session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Subscribe Topic3 Receipt Name Len : Length of the receipt name for subscription topic 3 , N9=[0,63] , 2 Bytes ; when N9=0 , the receipt for subscription topic 3 is not subscribed.

Subscribe Topic3 Receipt Name Data : Subscribe to Topic 3 Receipt Name Data, 63 Bytes . The first N9 bytes in Subscribe Topic3 Receipt Name Data are Subscribe Topic3 Receipt Name , and the remaining bytes are filled with 0 .

Message body example:

{ "id": "CP202910MxMQTTtest", "bv": "CP202910_V1.51", "sn": "\ufffd\ufffd4633\u0011", "name": "TagData", "version": "1.0", "timestamp": 60857, "payload": { "type": "TagData", "body": { "TagData": { "Ant": 1, "chennel": 0, "RSSI": 64796, "EPC": "E2000020340F013820406F61" } } } }

2.4.6 RFM_SET_GET_Publish_IOeven_TopicInfo (Set/Read Publish IO Event Topic Information)

- Format

| HEAD | Control Field |  |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Topic3 Info |  |
| 0xCF | 0xFF | 0x007F | 133，1Byte | 1Byte | 132 Bytes | 2Bytes |

- Response Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | Status | OPTION | Topic3 Info |  |
| 0xCF | 0xFF | 0x007F | 134 ， 1 Byte | 1 Byte | 1 Byte | 132 Bytes​ | 2 Bytes​ |

Parameter explanation:

Status : Command execution result status word, 1 Byte, can only be 0x00: success, other values are invalid.

Option : command control option, 0x01 is set, in the command format, it is followed by Topic4 Info , in the response format, it is not connected; 0x02 is read, in the response format, it is followed by Topic4 Info , in the command format, it is not connected; other values are invalid.

Topic4 Info : Topic 4 information, 132 Bytes , the specific composition is as follows:

| MSB LSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Publish Topic2 QoS | Publish Topic2 Name Len | Publish Topic2 Name Data | Subscribe Topic2 Receipt QoS | Subscribe Topic2 Receipt Name Len | Subscribe Topic2 Receipt Name Data |
| 1 Byte | N10, 2 Bytes | 63 Bytes​ | 1 Byte | N11, 2 Bytes | 63 Bytes​ |

Publish Topic4 QoS : Publish topic 4 session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Publish Topic4 Name Len : Length of the name of topic 4, N10=[0,63], 2 Bytes; when N10 = 0 , topic 4 is not published .

Publish Topic4 Name Data : Publish Topic 4 name data, 63 Bytes . The first N10 bytes of Publish Topic4 Name Data are Publish Topic4 Name , and the remaining bytes are filled with 0 .

Subscribe Topic4 Receipt QoS : Subscribe to topic 4 receipt session security level, 1 Byte .

0x00 : QoS0 ; 0x01 : QoS1 ; 0x02 : QoS2 ; other values are invalid.

Subscribe Topic4 Receipt Name Len : Length of the receipt name for subscription topic 4 , N11=[0,63] , 2 Bytes ; when N11=0 , the receipt for subscription topic 4 is not received.

Subscribe Topic4 Receipt Name Data : Subscribe to Topic 4 Receipt Name Data, N11 Bytes . The first N11 bytes in Subscribe Topic4 Receipt Name Data are Subscribe Topic4 Receipt Name , and the remaining bytes are filled with 0 .

Message body example:

{ "id": "CP202910MxMQTTtest", "bv": "CP202910_V1.51", "sn": "\ufffd\ufffd4633\u0011", "name": "TagData", "version": "1.0", "timestamp": 60857, "payload": { "type": "Trigger", "body": { "Trigger": {} } } }

Appendix A. Operational Status Returned by Tags

The operation status code returned by the tag is 8 bits in total, see Table A-1.

Table A-1 Operation status returned by the tag

| Operation status code | Operational Status | describe | Error Priority |
| --- | --- | --- | --- |
| 0x81 | Other Errors | Unknown error returned by tag |  |
| 0x82 | Storage area overflow | The target storage area does not exist |  |
| 0x83 | Storage Area Lock | Write operation to the memory area that is locked as non-writable Perform a read or erase operation on a storage area that is locked as unreadable. |  |
| 0x84 | Insufficient power | The tag does not have enough energy to complete the operation |  |
| 0x85 | Non-specific error | Unknown error returned by tag |  |

Appendix B. CRC16 checksum reference C code

as follows:

#define PRESET_VALUE 0xFFFF

#define POLYNOMIAL  0x8408

unsigned int uiCrc16Cal(unsigned char const  * pucY, unsigned char ucX)

{

unsigned char ucI,ucJ;

unsigned short int  uiCrcValue = PRESET_VALUE;

for(ucI = 0; ucI < ucX; ucI++)

{

uiCrcValue = uiCrcValue ^ *(pucY + ucI);

for(ucJ = 0; ucJ < 8; ucJ++)

{

if(uiCrcValue & 0x0001)

{

uiCrcValue = (uiCrcValue >> 1) ^ POLYNOMIAL;

}

else

{

uiCrcValue = (uiCrcValue >> 1);

}

}

}

return uiCrcValue;

}

Appendix C. Definition of STATUS.

Table A-6 STATUS definition

| STATUS | Error description |
| --- | --- |
| 0x00 | Execution successful (this only means that the module successfully received the tag response data. If there is a tag execution status in the tag response, it should be further determined whether the tag execution status is correct) |
| 0x01 | The parameter value is incorrect or out of range, or the module does not support the parameter value. |
| 0x02 | Command execution failed due to an internal module error |
| 0x03 | reserve |
| 0x12 | No inventory was performed to the tag or the entire inventory command was completed. |
| 0x14 | Tag response timeout |
| 0x15 | Demodulation tag response error |
| 0x1 6 | Protocol authentication failed |
| 0x1 7 | Wrong password |
| 0xFF | No more data |